ARM ISA

- Characteristics

- RISC aspects

  - Instructions have fixed length (32 bits)

  - Only load/store instructions can access memory

  - Arithmetic/logic instructions only operate on registers

- CISC aspects

  - Auto-increment/-decrement and PC-relative addressing modes

  - Condition codes (N, Z, V, C) are used for conditional execution of instructions

  - Multiple registers can be loaded/stored from/to a block of memory

- Memory organization

  - Byte-addressable, 32-bit address space

  - Supports little/big-endian

  - Supports data sizes of words (32), half words (16), bytes (8)

- Registers

  - 16 32-bit registers R0 – R15

  - R15 = PC; R12 = frame pointer (by convention); R13 = stack pointer; R14 = link register

  - CPSR/PSR = status register – stores conditional code flags

    - N = 1 if result is negative; cleared otherwise

    - Z = 1 if result is 0; cleared otherwise

    - V = 1 if overflow occurs; cleared otherwise

    - C = 1 if carry-out occurs; cleared otherwise

- Directives

- Label/EQU

  - LABEL: MOV R1, #1 (instruction at address 100)

  - Equivalent to LABEL EQU 100

  - EQU directive tells assembler to replace LABEL with 100

- ORIGIN

  - ORIGIN 200

  - Tells assembler to store subsequent instructions starting at address 200

- RESERVE

  - RESERVE 100

  - Tells assembler to reserve 100 bytes for data – no data will be loaded into these memory locations

- DATAWORD

  - DATAWORD 25

  - Number of entries in the list whose memory is being reserved

- Memory operations

  - LDR/STR – load/store words

  - LDRH/STRH – half words

  - LDRB/STRB – bytes

  - LDRSH/LDRSB – sign-extended loads for half words/bytes

  - LDM/STM{FD/EA}{cond} Rn{!}, reglist – multi-word load/store

    - {} means optional

    - Address mode

      - FD – increment address before access

      - EA – decrement address after access

    - Cond – conditional execution

    - ! – load final address into Rn (i.e. writeback)

    - reglist – one or more registers

- PC-relative addressing

  - LDR Rd, LABEL

    - i.e. Rd  [[PC] + offset(LABEL)]

  - PC += 8 when LDR executes

- Pre-index addressing

  - LDR Rd, [Rn, #8]

    - i.e. Rd  [[Rn] + 8] (indexed w/ offset)

  - LDR Rd, [Rn, #8]! – with writeback

    - Additionally increments Rn: Rn  [Rn] + 8

  - LDR Rd, [Rn, Rm, LSL #4]

    - Shifts [Rm] left by 4, then Rd  [[Rn]  [Rm]]

- Post-index addressing

  - LDR Rd, [Rn], #8

    - i.e. Rd  [[Rn]] (indexed w/o offset)

    - Also increments Rn: Rn  [Rn] + 8

  - LDR Rd, [Rn],  Rm, LSR #4

    - Rd  [[Rn]] then shifts [Rm] right by 4, and increments Rn: Rn  [[Rn]  [Rm]]

- Logical instructions

  - AND, ORR, EOR – bitwise and/or/xor

  - LSL/LSR/ASL/ASR Ri, Rj, #2

    - Ri  [Rj] shifted by 2 bits

    - Arithmetic shift preserves the sign in the MSB

    - Arithmetic shift/logical shift left – add 0s at LSB, push everything left (multiply by 2)

    - Logical shift right – add 0s at MSB, push everything right (divide by 2)

    - Arithmetic shift right – add 0s or 1s at MSB, depending on the sign

  - ROR Ri, Rj, #2

    - Ri  [Rj] rotated right by 2 bits

    - Rotate – copy bits from one end to another

    - C (carry) flag still retains the last bit being shifted out

    - E.g. C = 0, R1 = 0110…1011

      - ROR R1, R1, #2: C = 1, R1 = 110110…10

  - RRX Rj, Rj

    - Ri  [Rj] rotated right using C flag by 1 bit

    - E.g. C = 0, R1 = 0110…1011

      - RLC R1, R1, #2: C = 1, R1 = 00110…101

- Arithmetic instructions

  - ADD R0, R2, R4

    - R0  [R2] + [R4]

  - SUB R0, R1, R2, LSL #4 (left shift operand)

    - R0  [R1] + [R2] * 2^4

  - MUL R0, R1, R2

    - R0  [R1] * [R2]

  - MLA R0, R4, R5, R6

    - R0  [R4] * [R5] + [R6]

- Move instructions

  - MOV Rd, #value/Rm

    - Rd  value/[Rm]

  - MOV Rd, #value/Rm, LSL #shift

    - Equivalent to LSL Rd, Rm, #shift

  - MVN Rd, #value/Rm

    - Moves the bit complement of #value/[Rm]

- Test/compare instructions

  - TST Rn, #value/Rm

    - Performs bitwise AND then sets condition code

  - TEQ Rn, #value/Rm

    - Performs bitwise XOR then sets condition code

  - CMP Rn, Rm

    - [Rn] – R[m] then sets condition code

  - These instructions always update condition code

- Append S to opcode to update condition code

  - E.g. ADDS, ORRS

- Branch instructions

  - B{condition} LOC

    - Branches to LOC if condition code flags satisfy {condition}

  - BL LABEL – store PC in R14 (link register), branch to LABEL

  - BX LR – set PC to address stored in link register, i.e. reset return address

- Instructions encoding

  - RISC – instructions are limited to single words

  - 32-bit immediate must be assigned by:

    - Load 16-bit, shift left 16 bits, load 16-bit

    - Or save immediate in memory, then load from memory into register

  - Register-operand format

    - MSB – Rsrc1 – Rsrc2 – Rdst – opcode – LSB

  - Immediate-operand format

    - MSB – Rsrc – Rdst – immediate – opcode – LSB

  - Call format

    - MSB – immediate – opcode – LSB

- Condition codes

- The assembler

- Assembler directives – instruct the assembler to perform certain tasks during translation from source program  object program

  - AREA – specifies start of CODE/DATA

  - ENTRY – specifies start of program execution

  - DCD – label and initialize data operands

  - EQU – declare symbolic names for constants

  - RN – ??

- Pseudo-instructions

  - LDR Ri, =X

  - Where X is an address value

- Two-pass assembly

  - Generate symbol table – label  address (position relative of start of program)

  - Go through table, substitute symbol names with addresses

- For multiple files/modules

  - Assemble each module as individual program

  - Linker adds appropriate offsets to reflect where each module is in memory

  - Position-independent code – relative addressing, instructions etc.

  - Position-dependent code – references to external labels